Running a Full Node

Synergy Network

# 1. Overview

Running a full node on the Synergy Network enables you to independently verify the blockchain, propagate transactions, and support decentralized infrastructure.

# 2. Node Roles

- Full Node: Stores the complete blockchain, validates blocks, and relays data

- Validator Node: Participates in consensus and block production (must stake SYN and maintain uptime)

- Archive Node (optional): Maintains full historical state, useful for explorers and analytics

# 3. System Requirements

| Component | Minimum | Recommended |
| --- | --- | --- |
| CPU | 4 cores | 8+ cores |
| RAM | 16 GB | 32 GB |
| Storage | 500 GB SSD | 1 TB NVMe |
| Network | 100 Mbps symmetrical | 500 Mbps+ for validators |
| OS | Ubuntu 20.04+ / Debian 11+ | Linux-based server distro |

# 4. Software Requirements

- Docker & Docker Compose (or manual binary build)

- Go (for contract testing)

- Rust (for validator compilation)

- Git, curl, jq, systemd

# 5. Installation (Dockerized Setup)

git clone https://github.com/synergy-network/testnet.git
cd testnet
./setup.sh
./start-node.sh

# 6. Manual Installation

cargo build --release
./target/release/synergy-node start

# 7. Syncing the Chain

- Initial sync may take several hours

./scripts/apply-snapshot.sh
./check-sync.sh

# 8. Port Configuration

- Default P2P Port: 26656

- RPC: 26657

- WebSocket: 8546

Ensure ports are open in your firewall or VPS provider.

# 9. Monitoring Node Health

synergy-cli node status
tail -f ~/.synergy/logs/synergy-node.log

# 10. Keeping the Node Updated

git pull origin main
./update.sh

# 11. Common Issues

| Issue | Solution |
| --- | --- |
| Sync stuck | Check peers, reinit from snapshot |
| High CPU usage | Tune RocksDB config, use better hardware |
| RPC not reachable | Confirm firewall and port bindings |
| Out of disk space | Enable pruning or upgrade to larger drive |